IC Measurement Project Document

Project:

- Voltage/Current measurements

- Functionality measurements

Connections required:

Connect SCL and SDA pins of AD3 to Sensor

IC Part No: TCS34725

Address : x29 (Please refer the below table)

To access registers add x80 to the register address, to read what is in the clear data low address choose address as 0x94 (0x80 + 0x14)

I2C State Diagram:

I2C start is enabled every time you try  to access the register through I2C

Example 1: CDATAL and CDATAH (Clear Channel)

Choose address h29 in Waveforms window

Address as h94

Click on Read

Response should be h00

Example 2: AEN and WEN Command

Choose address h29 in Waveforms window

Address (Enable Register)

00001011 = h0B+h80 = h80

Click on Read

Response should be h00

Example 2: Device ID

Choose address h29 in Waveforms window

Address (ID Register) = h92 (h80 + h12)

Click on Read

Response should be h4D

Example 2: Read Blue Channel (Change the address value for different channels)

Choose address h29 in Waveforms window

Address (ID Register) = h9A (h80 + h1A) (BDATAL)

Address (ID Register) = h9B (h80 + h1A) (BDATAH)

Click on Read

Response should be h4

Notes: I2C 
S A R/W E

Write: SAWE

AW – 8 bits or 1 byte

Read: SARE

RGBC Operation

- RGBC gain control (AGAIN)

- RGBC Integration time (ATIME)

For example, if a 100-ms integration time is needed, the device needs to be programmed to:

256 − (100 / 2.4) = 256 − 42 = 214 = 0xD6

Conversely, the programmed value of 0xC0 would correspond to:

(256 − 0xC0) × 2.4 = 64 × 2.4 = 154 ms.

Control Register for Gain Control (0x0F)

Steps to connect the sensor

- Connect the correct AD3 device

  - Enumerate and getting no of devices available (FDwfEnum)

  - hdwf = C_int()

  - c_int(-1) choose the first device

- Power ON the AD3 (refer AnalogI).py) or dwf constants

- Questions: Check the Gain, Check the ATIME

dwf. FDwfAnalogIOReset(hdwf)

dwf. FDwfAnalogI0Channe1NodeSet(hdwf, c_int(®), c_int(1), _double(5.0)) dwf. FDwfAnalogIOChannelNodeSet(hdwf, c_int(e), c_int(0), c_double(1)) dwf. FDwfAnalogI0ChannelNodeSet(hdwf, c_int(1), c_int(1), c_double(-5.0)) dwf. FDwfAnalogI0ChannelNodeSet(hdwf, c_int(1), c_int(0), c_double (1))

dwf. FDwfAnalogI0EnableSet(hdwf, c_int(1))

time. sleep(0.5)

vpos = _double()

vneg = _double()

dwf. FDwfAnalogI0Status(hdwf)

dwf. FDwfAnalogI0ChannelNodeStatus(hdwf, c_int(8), c_int(1), byref(vpos)) dwf. FDwfAnalogI0ChannelNodeStatus(hdwf, c_int(1), c_int(1), byref (vneg))

print(f"Power: +(vpos. value:.2f}V, {vneg.value: .2f)V")

def write_register(reg, value) :

iNak = c_int()

rgTX = (c_ubyte * 2) (reg, value)

dwf. FDwfDigitalI2cWrite(hdwf, c_int(DEVICE_ADDR), rgTX, c_int(2), byref (iNak))

return iNak.value == 0

def read_register (reg):

iNak = c_int()

rgTX = (c_ubyte * 1) (reg)

dwf. FDwfDigitalI2cWrite(hdwf, c_int(DEVICE_ADDR), rgTX, c_int(1), byref (iNak))

if iNak. value == 0:

rgRX = (c_ubyte * 1) ()

dwf. FDwfDigitalI2cRead(hdwf, c_int(DEVICE_ADDR), rgRX, c_int(1), byref (iNak))

if iNak.value == 0:

return rgRx[0]

return None

dwf. FDwfDigital12cRateSet(hdwf, _double(108e3))

dwf. FDwfDigital12cSc1Set(hdwf, c_int(0)) dwf.FDwfDigitalI2cSdaSet(hdwf, c_int(1))

iNak = c_int()